Practical-3

AIM: Identify various computer network tools:

Computer network tools includes physical topologies, structured cabling types and diagnostic command line utilities.

- Various Network Topologies.

- Star Topology: All devices connected to a central hub or switch, if one cable fails, only that device is disconnected.

- Bus Topology: Devices connect to a single central backbone cable; a break in the main cable brings down the entire network.

- Ring Topology: Device form a circular data path; data travels in one direction, and a break  can disrupt the loop.

- Mesh Topology: Every device connects to multiple or all other devices; highly redundant and reliable.

- Tree Topology: Combines star and bus topologies in a hierarchical parent-child structure.

- Hybrid Topology: A mix of two or more basic topologies(like the Star-Ring) to fit large space.

2. Types of Network Cables & Crimping Implementation

- Twisted Pair Cable (UTP/STP): Unshielded or shielded copper wires twisted to reduce signal interference; uses RJ-45 connectors.

- Coaxial Cable: Central copper conductor wrapped in insulation and a metal shield; used for cable television and older LANs.

- Fiber Optic Cable: Glass threads transmitting data via light pulses; supports very high speeds over long distances.

- Straight-Through Cable: Both ends use the T568B standard (or both T568A). Used to connect different devices (PC to Switch/Hub).

- Cross-Wired (Crossover) Cable: One end uses T568A and the other uses T568B (TX and RX lines cross). Used to connect similar devices directly (PC to PC, Switch to Switch).

- Untwist and align wires: Untwist the pairs and arrange them neatly side by side based on standard colour codes (T568A or T568B).

- Insert into RJ-45: Carefully push the wires into the RJ-45 connector, ensuring each colour touches the pin terminals at the front.

3. Basic Network Commands

- ping [IP/URL]: Tests network connectivity and response time to a target host.

- ipconfig (Windows) / if config (Linux): Displays current IP address, subnet mask, and default gateway configurations.

- tracert (Windows) / traceroute (Linux): Traces the exact network path and packet hops taken to reach a destination.

- nslookup: Resolves a domain name to its corresponding IP address via DNS lookup.

- netstat: Displays active network connections, routing tables, and interface statistics.

- arp -a: Displays the Address Resolution Protocol cache mapping IP addresses to physical MAC addresses on your local network.

- route print: Displays the local routing table to show exactly how your computer routes outbound network traffic.

- ipconfig : Empties and resets the local DNS resolver cache to clear out corrupted or outdated domain lookups.